LadderStudio

Complete Ladder Block Catalog

Technical reference generated from ladder_blocks.py (BLOCK_DEFS)

122 blocks across 20 categories

July 2026

# Table of Contents

# Overview

This document lists all 122 blocks defined in BLOCK_DEFS (ladder_blocks.py), organized by functional category. For each block, the document indicates the positional arguments (in the order they appear in the rung), the display name, and a brief description of how it works, based on both the block's visual definition and the configuration fields (specs) used by the editor.

This is an expanded version of the initial survey (which covered Sensors, Communication, RF 433, RS-485, Infrared, EEPROM, Motor Control, and Motion) to also include: Timers/Counters, Comparators, Math, Trigonometry, Bitwise Logic, Data Handling, Sequencers/Flow Control, HMI/LCD, 7-Segment Display, RTC, and the End-of-Scan marker.

# 1. Timers and Counters

Classic PLC timer and counter blocks (compatible with the Allen-Bradley/RSLogix standard). Each one keeps an accumulated value (Accum) that evolves every scan until it reaches the Preset.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| TON | Timer, Base, Preset | Timer On-Delay | Turns the output (DN) on only after the input has stayed on for a time equal to the Preset. If the input turns off before that, the accumulator resets. |
| TOF | Timer, Base, Preset | Timer Off-Delay | Keeps the output on for a time (Preset) after the input turns off. Useful for 'delaying shutdown' (e.g., a fan that keeps running after the motor stops). |
| TP | Timer, Base, Preset | Pulse Timer | Generates a fixed-duration pulse (Preset) from an input edge, regardless of how long the input stays on. |
| RTO | Timer, Base, Preset (+Reset) | Retentive On-Delay | Like TON, but the accumulator is retentive: it doesn't reset when the input drops, only when a dedicated RESET input is triggered. |
| CTU | Counter, Preset (+Reset) | Count Up | Increments the accumulator on every rising edge at the CU input. Activates DN when the accumulator reaches the Preset. RESET clears the counter. |
| CTD | Counter, Preset (+Reset) | Count Down | Decrements the accumulator on every pulse at CD. Useful for countdowns (e.g., remaining parts). |
| CTC | Counter, Preset | Circular Counter | Cyclic counter: once it reaches the Preset, it automatically wraps back to zero and keeps counting (no external reset needed). |
| CTUD | Counter, Preset (+CU/CD/Reset) | Count Up-Down | Combines CTU and CTD in a single block: one input increments, another decrements, and a third resets the accumulator. |

# 2. Comparators

Compare two (or three) numeric values and energize the rung's output when the logical condition is true. Typically used before a coil or as a rung condition.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| EQU | Source A, Source B | Equal (A = B) | True when A is exactly equal to B. |
| NEQ | Source A, Source B | Not Equal (A ≠ B) | True when A is different from B. |
| LES | Source A, Source B | Less Than (A < B) | True when A is strictly less than B. |
| LEQ | Source A, Source B | Less Than or Equal (A ≤ B) | True when A is less than or equal to B. |
| GRT | Source A, Source B | Greater Than (A > B) | True when A is strictly greater than B. |
| GEQ | Source A, Source B | Greater Than or Equal (A ≥ B) | True when A is greater than or equal to B. |
| LIM | Low Limit, Source, High Limit | Limit Test | True when the Source value falls within the range between Low Limit and High Limit (tolerance window). |
| MEQ | Source, Mask, Compare | Masked Equal | Applies a bit mask to Source before comparing it to Compare — allows testing only selected bits while ignoring the rest. |
| HYST | In, Lo, Hi | Hysteresis | Comparator with a dead band: turns on above the Hi threshold and only turns off below the Lo threshold, preventing chattering near the switching point. |

# 3. Math

Basic arithmetic operations on numeric variables (integer or floating-point, depending on the tag type used).

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| ADD | Dest, Src A, Src B | Addition | Destination = Src A + Src B. |
| SUB | Dest, Src A, Src B | Subtraction | Destination = Src A − Src B. |
| MUL | Dest, Src A, Src B | Multiplication | Destination = Src A × Src B. |
| DIV | Dest, Src A, Src B | Division | Destination = Src A ÷ Src B. |
| ABS | Dest, Src x | Absolute Value | Destination = ¦x¦ (removes the negative sign). |
| SQR | Dest, Src x | Square Root | Destination = √x. |
| XPY | Dest, Src x, Src y | Power | Destination = x raised to the power y (x^y). |
| NEG | Dest, Src x | Negate | Destination = −x (flips the sign). |
| INC | Source | Increment | Increments the variable by 1 (Source = Source + 1). |
| DEC | Source | Decrement | Decrements the variable by 1 (Source = Source − 1). |
| CLR | Source | Clear | Resets the variable's value to zero. |
| MOV | Source, Dest | Move | Copies the value of Source to Destination (without changing the source). |

# 4. Trigonometry and Advanced Functions

Floating-point math functions (use LUTs or floating-point libraries in the generated C code, depending on the target microcontroller).

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| SIN | Dest, Src x | Sine | Destination = sin(x), x in radians. |
| COS | Dest, Src x | Cosine | Destination = cos(x), x in radians. |
| TAN | Dest, Src x | Tangent | Destination = tan(x), x in radians. |
| ASN | Dest, Src x | Arc Sine | Destination = asin(x), result in radians. |
| ACS | Dest, Src x | Arc Cosine | Destination = acos(x), result in radians. |
| ATN | Dest, Src x | Arc Tangent | Destination = atan(x), result in radians. |
| LN | Dest, Src x | Natural Logarithm | Destination = ln(x) (base-e log). |
| LOG | Dest, Src x | Base-10 Logarithm | Destination = log10(x). |
| DEG | Dest, Src x | Radians to Degrees | Converts an angle from radians to degrees. |
| RAD | Dest, Src x | Degrees to Radians | Converts an angle from degrees to radians. |

# 5. Bitwise Logic

Classic boolean operations applied word-by-word (bitwise), useful for manipulating flags, masks, and I/O registers.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| AND | Dest, Src A, Src B | Bitwise AND (A & B) | Each destination bit is 1 only if the corresponding bit is 1 in both A and B. |
| OR | Dest, Src A, Src B | Bitwise OR (A ¦ B) | Each destination bit is 1 if the corresponding bit is 1 in A or in B. |
| NOT | Dest, Src A | Bitwise NOT (¬A) | Inverts all bits of A (bitwise complement). |
| XOR | Dest, Src A, Src B | Bitwise XOR (A ^ B) | Each destination bit is 1 only if A and B differ at that bit. |

# 6. Data Handling (Shift / Files)

Blocks for shifting bits, rotating registers, copying/filling memory blocks, and scaling values — equivalent to the file instructions found in industrial PLCs.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| BSL | File, Control, Source Bit, Length | Bit Shift Left | Shifts all bits of the File array one position to the left, inserting the Source Bit at the end. |
| BSR | File, Control, Source Bit, Length | Bit Shift Right | Same as BSL, but shifts to the right. |
| ROL | File, Control, Rotate Number | Rotate Left | Rotates the register's bits to the left (the bit that exits one side re-enters on the other, with no loss). |
| ROR | File, Control, Rotate Number | Rotate Right | Same as ROL, but rotating to the right. |
| COP | Source, Destination, Length | Copy File | Copies a block of Length elements from Source to Destination. |
| FLL | Source, Destination, Length | Fill File | Fills Length elements of Destination with the same value as Source. |
| MVM | Destination, Source, Mask | Masked Move | Copies Source to Destination, but only for the bits enabled by the Mask (the remaining destination bits stay unchanged). |
| SCL | Input, Rate, Offset, Output | Linear Scale | Output = Input × Rate + Offset — simple linear scaling conversion (e.g., ADC counts to engineering units). |
| SCP | Input, In Min, In Max, Out Min, Out Max, Output | Scale with Parameters | Maps Input from an input range (In Min–In Max) to an output range (Out Min–Out Max), interpolating linearly. |

# 7. Sequencers and Flow Control

Output-sequencing blocks and program-flow jump instructions for the Ladder program.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| SQO | File, Mask, Destination, Control, Length, Position | Sequencer Output | On each trigger, advances to the next position in File and transfers the (masked) bit pattern to Destination — used for step sequences (e.g., traffic light, conveyor). |
| SQC | File, Mask, Source, Control, Length, Position | Sequencer Compare | Compares the masked Source against the current File position; energizes the output if they match — used to check for an expected sequence. |
| SQL | File, Source, Control, Length, Position | Sequencer Load | Records (writes) the value of Source into the current File position on each trigger — used to 'teach'/record a sequence. |
| JMP | Reference Bit / Label | Jump | Diverts scan execution directly to the indicated label (LBL), skipping the rungs in between. |
| JSR | Subroutine | Jump to Subroutine | Diverts execution to a named subroutine and returns to the call point once it finishes. |
| LBL | Reference Bit / Label | Label Location | Marks a destination point in the program for JMP instructions. |
| TND | — | Temporary End | Ends the scan at that point in the program, ignoring the remaining rungs until the next cycle. |
| IIM | Input Word, Mask | Immediate Input Mask | Immediately reads (without waiting for the next scan) the physical state of the inputs, applying a bit mask. |
| IOM | Output Word, Mask | Immediate Output Mask | Immediately writes the bits enabled by the mask to the physical outputs, without waiting for the end of the scan. |
| REF | — | I/O Refresh | Forces an immediate update of all physical inputs/outputs in the middle of the scan. |
| TCY | Timer, Base, T.ON, T.OFF | Cyclic Timer | Generates an oscillating (square-wave) signal, automatically alternating between on-time (T.ON) and off-time (T.OFF) — useful for blinking outputs or scan clocks. |
| STEPPER | Speed, Mode, Pins, Operation, Steps, Hold/Coast, Done, Invert, Ramp | Stepper Motor Control | Generates the pulse sequence (STEP-DIR, full-wave, half-step, or bipolar) to move a stepper motor a set number of steps or continuously, with optional acceleration/deceleration ramp. |

# 8. HMI / LCD Display

Human-machine interface blocks for character LCD displays (16x2/20x4, via 4-bit parallel, I2C, or SPI 74HC595).

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| LCD_INIT | Columns, Rows, Interface | Initialize LCD | Configures the display size (e.g., 16x2) and the physical connection mode (4-bit parallel, I2C expander, or SPI 595 shift register). |
| LCD_STR | Row, Column, Text | Write Fixed Text | Writes a literal fixed string at a given position (row/column) on the display. |
| LCD_VAL | Row, Column, Source, Decimal Places | Write Numeric Value | Formats and displays the value of a numeric variable at the given position, with a configurable number of decimal places. |
| LCD_SCROLL | Row, Speed, Text | Scrolling Text | Makes text longer than the display scroll horizontally on the given row, at the rate defined by the speed. |
| LCD_ICON | Slot, Pixel Matrix | Custom Icon (CGRAM) | Writes a custom graphic character (5x8 pixel matrix) to a slot in the LCD controller's CGRAM memory, for use as an icon. |
| LCD_BAR | Position, Length, Current Variable, CGRAM Slot, Maximum Value | Progress Bar | Draws a graphical progress bar (using custom characters) proportional to the current value over the maximum value. |
| LCD_PWR | State (ON/OFF) | Turn Display On/Off | Turns the LCD's backlight/display on or off without erasing the content stored in memory. |
| LCD_IOMAP | Position, Base Tag, Bit Count, OFF Char, ON Char | Adaptive I/O Monitor | Visually displays the state of several consecutive bits (e.g., inputs or outputs) as a sequence of on/off characters. |
| LCD_TIME | Position, Total Seconds | HH:MM:SS Clock | Converts a value in total seconds and displays it in Hours:Minutes:Seconds clock format. |
| LCD_TEMP | Position, Variable, Width | Display Temperature | Displays a formatted temperature value (with symbol/unit) at the given position. |
| LCD_CLIMA | Position, Temp, Humidity | Display Temp + Humidity | Displays temperature and humidity together on a single line, typically coming from a SENS_CLIMA sensor. |
| LCD_FORMAT | Position, Text with Tags | Dynamic Formatted Text | Lets you assemble a line combining fixed text and embedded variables (e.g., 'Temp: {N:0}'), like a simplified printf. |
| LCD_MENU | Current Screen, Key, Max Screens | Menu Navigation | Controls switching between 'screens' (pages) of a menu on the LCD, advancing based on the key pressed, up to the configured maximum number of screens. |
| IHM_CTRL | Button +, Button −, Button Enter, Max Screens, Base Tag, Enter Output | HMI Navigation Control | Combined block that reads the physical navigation buttons (up/down/confirm) and controls the current screen/menu index, pulsing an output when confirmed. |
| KEYPAD_READ | Destination, Matrix Configuration | Matrix Keypad Reading | Scans a matrix keypad (e.g., 4x3 or 4x4) and writes the pressed key to the destination variable. |
| LCD_CLR | Row (-1 = all) | Clear Display | Erases the content of a specific LCD row, or all rows if the value is -1. |

# 9. 7-Segment Display

Blocks for controlling numeric 7-segment displays, with support for multiple drivers (direct drive, 74HC595 shift register, TM1638, MAX7219, or I2C expander).

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| SEG7_INIT | Driver, Digits | Configure Display | Sets the hardware driver type used (DIRECT/SPI_595/TM1638/MAX7219/I2C) and how many digits the display has. |
| SEG7_STR | Text, Position | Write Letter/Symbol | Writes a simple alphanumeric character (approximated on the available segments) at a display position. |
| SEG7_VAL | Source, Position, Width | Write Numeric Value | Converts and displays a numeric value from the Source variable, occupying the specified width (number of digits) starting at the given position. |
| SEG7_CLR | — | Clear Display | Clears all digits on the 7-segment display. |

# 10. RTC and Time Management

Blocks for real-time clock (DS3231 RTC) and time-based scheduling.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| RTC_READ | Dest Hour, Dest Min, Dest Sec | Read RTC | Reads the current time from the DS3231 RTC module (via I2C) and writes hours/minutes/seconds to the destination variables. |
| RTC_WRITE | Hour, Minute, Second | Set RTC | Writes a new time to the DS3231 RTC module, from fixed values or variables. |
| PARSE_TIME | Source String, Dest Hour, Dest Min | Parse Time from Text | Parses a received string (e.g., via NTP/Wi-Fi/Bluetooth) and extracts the hour and minute fields from it. |
| AGENDA | DN, Output ID, BT String, Current Hour/Min, On H/M, Off H/M | Hybrid IoT Schedule | Automatically turns an output on and off at programmed times (fixed or received via Bluetooth/string), comparing them against the RTC's current hour/minute. |

# 11. Sensors

Blocks for reading analog and digital temperature/climate sensors.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| SENS_TEMP | Model, Pin, Destination | Temperature Sensor | Reads a temperature sensor (analog LM35 or digital DS18B20 over the 1-Wire protocol) and writes the value (fixed-point x10) to the destination variable. |
| SENS_CLIMA | Model, Pin, Dest Temp, Dest Humidity | Temperature and Humidity Sensor | Reads a combined sensor (DHT11 or DHT22) over its own digital protocol and writes temperature and humidity to separate variables. |

# 12. Communication: Wi-Fi, Bluetooth, MQTT, NTP

Wireless connectivity and network protocol blocks, built around the ESP01 (Wi-Fi) and HC-05 (Bluetooth) modules and IoT/cloud-related firmware features.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| WIFI_INIT | TX, RX, Baud | Initialize Wi-Fi (ESP01) | Configures the serial pins (TX/RX) and baud rate used to communicate with the ESP01 Wi-Fi module. |
| WIFI_SEND | Type, Data | Send Data over Wi-Fi | Sends fixed text or the value of a variable to the Wi-Fi module, typically forwarded to a server/broker. |
| WIFI_RECV | Destination | Receive Byte over Wi-Fi | Reads a single byte received from the Wi-Fi module's serial buffer and writes it to the destination variable. |
| WIFI_RECV_STR | Destination, Buffer Size | Receive Text over Wi-Fi | Reads a full string received from the Wi-Fi module (up to the configured buffer size) and writes it to a string variable. |
| BT_INIT | TX, RX, Baud | Initialize Bluetooth (HC-05) | Configures the serial pins and baud rate for the HC-05 Bluetooth module. |
| BT_SEND | Type, Data | Send Data over Bluetooth | Sends fixed text or the value of a variable over the Bluetooth connection (e.g., to a phone app). |
| BT_RECV | Destination | Receive Byte over Bluetooth | Reads a single byte received from the Bluetooth buffer and writes it to the destination variable. |
| BT_RECV_STR | Destination, Buffer Size | Receive Text over Bluetooth | Reads a full string received over Bluetooth and writes it to a string variable. |
| MQTT_PUB | Topic, Value | MQTT Publish | Publishes a value (or text) to an MQTT topic, to be consumed by an IoT broker/server. |
| MQTT_SUB | Topic, Destination | MQTT Subscribe | Subscribes to an MQTT topic and writes the received value (numeric, or a coil trigger) to the destination variable. |
| NTP_SYNC | Dest Hour, Dest Min, Dest Sec, OK | NTP Time Sync | Queries an internet time server (NTP) over Wi-Fi and writes the correct hour/minute/second, pulsing a confirmation output (OK) on success. |
| WIFI_FACTORY_RESET | — | Reset Wi-Fi | Forces the ESP01 module back into its initial configuration mode (config portal), erasing saved credentials. |

# 13. Analog Output (PWM)

Analog signal generation via pulse-width modulation.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| ANALOG_OUT_PWM | Duty Cycle (%), Output | PWM Analog Output | Generates a PWM signal with a duty cycle from 0 to 100% on an output, simulating an average analog voltage level (e.g., for brightness/speed control). |

# 14. Motion

Blocks aimed at motion control: smooth ramps, speed measurement, and position reading via encoder.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| RAMP | Output, Target, Time, Base | Ramp / Soft-Start | Makes an output (e.g., PWM) rise or fall smoothly to the Target value over the specified Time, instead of jumping instantly — avoids abrupt starts/stops. |
| TACH | Output, Sensor, Window, Pulses/Rev, Unit, Circumference | Tachometer / RPM Measurement | Counts pulses from a sensor (e.g., encoder or inductive sensor) within a time window and calculates rotational speed (RPM), frequency (Hz), or linear speed (m/s, m/min). |
| ENC | Count, Phase A, Phase B, Reset | Quadrature Encoder | Decodes the quadrature signals (Phase A/Phase B) of an incremental encoder to accumulate position/count, with direction of rotation detected from the phase offset; Reset clears the count. |

# 15. RF 433 MHz

Short-range wireless communication via 433 MHz radio-frequency modules, including rolling-code support for secure remote controls.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| RF433_TX | TX Pin, Data, Password | Transmit RF 433 | Transmits data (e.g., a command code) over 433 MHz radio, along with a simple validation password. |
| RF433_RX | RX Pin, Destination, Dest. Password | Receive RF 433 | Receives a 433 MHz radio packet and writes the received data and password to variables, for later validation. |
| RF433_SEC_RX | RX Pin, Dest Button, OK, EEPROM, Master Key | Receive Secure RF 433 (Rolling Code) | Receives and validates a rolling code from a secure remote control, using a master key and a counter saved in EEPROM to prevent replay of a captured signal. |
| RF433_SEC_TX | TX Pin, Button Number, Master Key, Serial | Transmit Secure RF 433 TX | Transmits an encrypted rolling-code command, identified by button number and remote serial number, using the same master key as the receiver. |
| RF433_LEARN | RX Pin, Confirm, Serial Read, Status Read | RF Learning Mode | Puts the system into 'learn' mode for a new remote control: captures the serial number of the presented remote and saves it (upon confirmation) for future use. |

# 16. RS-485 (Field Network)

Industrial multi-drop serial bus (master/slave), typically used to link multiple PLC modules on a field network.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| RS485_SLAVE_NODE | ID, Baud, TX, DIR, Commanded Bits, Status Outputs, Status Inputs | RS-485 Slave Node | Configures the device as an addressable slave node (ID from 1 to 247) on the RS-485 network, exposing output bits controlled by the master and reporting input/output status. |
| RS485_MASTER_CFG | Baud, DIR, TX, RX | RS-485 Master Configuration | Configures the device as the master of the RS-485 network, setting the baud rate and the direction-control pin (half-duplex DE/RE). |
| RS485_CMD | Target ID, CMD, Data, Status Destination, OK | Send RS-485 Command | The master sends a command with associated data to a specific slave node (Target ID) and receives back the status, pulsing OK on success. |

# 17. Infrared (IR)

Transmission and reception of infrared signals (remote controls), supporting the NEC, RC5, SONY protocols and a generic 'AC' mode for custom timings.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| IR_TX | TX Pin, Data, Protocol, Hex Base, AC Timings, AC Data | Transmit IR | Transmits an IR code modulated at 38kHz, following the chosen protocol (NEC/RC5) or fully custom timings (AC mode, for example for air-conditioner remotes). |
| IR_RX | RX Pin, Destination, Protocol, Hex Base | Receive IR | Receives and decodes a 38kHz IR signal according to the configured protocol, writing the decoded code to the destination variable. |

# 18. EEPROM

Non-volatile variable persistence, whether in the microcontroller's internal EEPROM or external I2C/SPI modules.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| EEP_WRITE | Variable, Destination, Model, Slots (Protection Factor) | Save to EEPROM | Writes a variable's value to EEPROM (internal, or external I2C/SPI models such as 24C32/24C256), using multiple rotating 'slots' to reduce write wear (wear leveling). |
| EEP_READ | Destination Variable, Source, Model, Slots | Read from EEPROM | Reads the previously saved value from EEPROM and writes it to the destination variable, using the same protection-slot scheme. |

# 19. Motor and Relay Control

Safety logic blocks for driving motors and contactors, with built-in electrical interlocking.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| SELO | On, Off, Output, Priority | Seal-In / Self-Hold Command | Implements the classic seal-in (self-hold on/off) logic: a pulse on 'On' energizes the Output, which stays energized until a pulse on 'Off'; the Priority parameter defines which command wins if both occur at the same time. |
| MOTOR_REV | Left On, Off, Right On, Out K1, Out K2 | Interlocked Reversal | Controls two contactors (K1/K2) to spin a motor in two directions, with electrical/logical interlocking that prevents both contactors from closing at the same time (phase short-circuit). |
| ESTRELA_TRIANGULO | On, Off, K1, K3, K2, Time | Star-Delta Starter | Runs the classic three-phase motor start sequence: starts in star (K1+K3) to reduce inrush current and, after the configured Time, switches to delta (K1+K2) for running mode, with interlocking between K2 and K3. |

# 20. End of Scan

Mandatory structural marker for the end of the Ladder program.

| Block | Args | Name | Description / How it works |
| --- | --- | --- | --- |
| END | — | End of Scan | Marks the logical end of the Ladder program; anything after this block is not executed in the normal scan. Referenced in the code as the SYS_FIRST_SCAN/anti-flicker mechanism for the LCD. |