LadderStudio

Supported Chip Catalog by Family and Architecture

Technical reference generated from the database/ package (CHIP_SPECS + EEPROM_SPECS)

53 microcontroller targets across 10 families, plus an appendix of 21 external EEPROM parts

July 2026

# Table of Contents

# Overview

This document lists every microcontroller target defined in LadderStudio's database/ package (the merged CHIP_SPECS dictionary), organized first by silicon family (e.g., PIC, AVR, STM8) and then by internal sub-architecture (e.g., PIC12F/PIC16F/PIC18F, ATmega/ATtiny). For each chip, the table shows package, pin count, RAM, Flash/ROM size, the hardware peripherals flagged in its spec (ADC, PWM, UART, I2C, SPI, CAN, Wi-Fi), and a short note on anything distinctive about that part.

In total, the database currently defines 53 microcontroller targets across 10 silicon families. An appendix at the end lists the 21 external EEPROM memory chips (I2C 24Cxx and SPI 25LCxx series) supported by the EEP_WRITE/EEP_READ ladder blocks — these are memory devices, not code-execution targets, so they are kept separate from the main chip families.

# 1. Microchip PIC (8-bit)

Microchip's classic 8-bit PIC core, LadderStudio's original and most mature backend. Split into three sub-architectures of increasing capability.

## 1.1 PIC12F — Baseline / Enhanced Midrange (8-pin)

Tiny 8-pin devices for the simplest projects. No LATx registers on the oldest parts; limited I/O (5–6 usable pins) and small RAM/Flash.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| PIC12F629 | DIP | 8 | 64 B | 1 KB | — | No ADC. Baseline core. |
| PIC12F675 | DIP | 8 | 64 B | 1 KB | ADC | Baseline core with analog input. |
| PIC12F683 | DIP | 8 | 128 B | 2 KB | ADC, PWM | Enhanced baseline with CCP module. |
| PIC12F1572 | DIP | 8 | 256 B | 2 KB | ADC, PWM, UART | Enhanced midrange core (14-bit), on-board EUSART. |
| PIC12F1822 | DIP | 8 | 128 B | 2 KB | ADC, PWM, UART, I2C, SPI | Full MSSP (shared I2C/SPI) despite the small pin count. |
| PIC12F1840 | DIP | 8 | 256 B | 4 KB | ADC, PWM, UART, I2C, SPI | Largest 12F in the set; same peripheral mix as the 1822. |

## 1.2 PIC16F — Midrange (14/18/28/40-pin)

The workhorse midrange family, spanning from bare-bones comparator chips to full-featured 40-pin devices with ADC, PWM, UART, I2C, and SPI.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| PIC16F630 | DIP | 14 | 64 B | 1 KB | — | No ADC; comparator-oriented baseline part. |
| PIC16F676 | DIP | 14 | 64 B | 1 KB | ADC | Same family as 630, with analog input added. |
| PIC16F716 | DIP | 18 | 128 B | 2 KB | ADC, PWM | Entry-level with CCP/PWM. |
| PIC16F628A | DIP | 18 | 224 B | 2 KB | PWM, UART | Very popular no-ADC part with USART, common in hobbyist designs. |
| PIC16F88 | DIP | 18 | 368 B | 4 KB | ADC, PWM, UART, I2C, SPI | Full peripheral set in an 18-pin package. |
| PIC16F1827 | DIP | 18 | 384 B | 4 KB | ADC, PWM, UART, I2C, SPI | Enhanced midrange (14-bit core) successor to the 88/628. |
| PIC16F72A | DIP | 28 | 128 B | 2 KB | ADC, PWM, UART, I2C, SPI | 28-pin full-peripheral midrange part. |
| PIC16F74 | DIP | 40 | 192 B | 4 KB | ADC, PWM, UART, I2C, SPI | 40-pin variant with more I/O. |
| PIC16F877A | DIP | 40 | 368 B | 8 KB | ADC, PWM, UART, I2C, SPI | The best-known PIC of all — the reference 40-pin, full-peripheral chip. |

## 1.3 PIC18F — High-End 8-bit (28/40-pin)

Higher performance 8-bit core with deeper stack, more RAM/Flash, and (on these two parts) native USB.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| PIC18F2550 | DIP | 28 | 2 KB | 32 KB | ADC, PWM, UART, I2C, SPI | 28-pin part with native USB support. |
| PIC18F4550 | DIP | 40 | 2 KB | 32 KB | ADC, PWM, UART, I2C, SPI | 40-pin sibling of the 2550, same USB-capable core. |

# 2. Microchip PIC24 (16-bit)

16-bit successor to the PIC line, offering wider registers, more RAM/Flash headroom, and higher throughput for more demanding ladder programs.

## 2.1 PIC24F

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| PIC24FJ64GA002 | DIP | 28 | 8 KB | 64 KB | ADC, PWM, UART, I2C, SPI | 28-pin entry point into the 16-bit PIC24 family. |
| PIC24FJ128GB106-I/PT | TQFP | 64 | 16 KB | 128 KB | ADC, PWM, UART, I2C, SPI | 64-pin, large RAM/Flash — the biggest PIC24 target in the database. |

# 3. Atmel / Microchip AVR (8-bit RISC)

RISC 8-bit core with single-cycle instruction execution. Split between the full-featured ATmega line and the small-footprint, low-pin-count ATtiny line.

## 3.1 ATmega

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ATmega168 | DIP | 28 | 1 KB | 16 KB | ADC, PWM, UART, I2C, SPI | Entry-level ATmega, predecessor of the 328P. |
| ATmega328P | DIP | 28 | 2 KB | 32 KB | ADC, PWM, UART, I2C, SPI | The Arduino Uno chip — the most common AVR target. |
| ATmega32 | TQFP | 44 | 2 KB | 32 KB | ADC, PWM, UART, I2C, SPI | Classic 32 KB AVR, popular in industrial/educational designs. |
| ATmega32A | DIP | 40 | 2 KB | 32 KB | ADC, PWM, UART, I2C, SPI | DIP version of the ATmega32 line. |
| ATmega32U4 | TQFP | 44 | 2.5 KB | 32 KB | ADC, PWM, UART, I2C, SPI | Native USB core (Arduino Leonardo/Micro chip). |
| ATmega1284P | DIP | 40 | 16 KB | 128 KB | ADC, PWM, UART, I2C, SPI | Large-memory 40-pin AVR for bigger ladder programs. |
| ATmega2560 | TQFP | 100 | 8 KB | 256 KB | ADC, PWM, UART, I2C, SPI | The Arduino Mega chip — most I/O and Flash in the AVR set. |
| ATmega8515 | TQFP | 44 | 512 B | 8 KB | PWM, UART, SPI | No built-in ADC; external-memory-bus-oriented legacy part. |

## 3.2 ATtiny

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ATtiny13V | DIP | 8 | 64 B | 1 KB | ADC, PWM, I2C, SPI | Smallest AVR in the database, 8-pin, no UART. |
| ATtiny85 | DIP | 8 | 512 B | 8 KB | ADC, PWM, I2C, SPI | Popular 8-pin ATtiny with more RAM/Flash than the 13V. |

# 4. STMicroelectronics STM8 (8-bit)

STMicroelectronics' 8-bit core, organized in three product lines of increasing peripheral count and memory size.

## 4.1 STM8S Value Line

Cost-optimized entry line.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| STM8S003F3 | TSSOP | 20 | 1 KB | 8 KB | ADC, PWM, UART, I2C, SPI | Smallest, cheapest STM8S Value Line part. |
| STM8S005C6 | LQFP | 48 | 2 KB | 32 KB | ADC, PWM, UART, I2C, SPI | Larger-pin-count Value Line part with more Flash. |

## 4.2 STM8S Access Line

General-purpose mid-tier line.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| STM8S103F3 | TSSOP | 20 | 1 KB | 8 KB | ADC, PWM, UART, I2C, SPI | Extremely popular low-cost STM8 (widely used in "STM8S103F3 breakout" boards). |
| STM8S105K4 | LQFP | 32 | 2 KB | 16 KB | ADC, PWM, UART, I2C, SPI | Larger Access Line part with more Flash. |

## 4.3 STM8S High-Density

Top-tier STM8S line with the most memory, timers, and I/O.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| STM8S207MB | LQFP | 80 | 6 KB | 128 KB | ADC, PWM, UART, I2C, SPI, CAN | The only STM8 in the set with CAN — targets industrial/automotive designs. |

# 5. Freescale / NXP 68HC(S)08 and 68HC11

Legacy Freescale (now NXP) 8-bit architectures: the older 68HC11 core and its successor, the HCS08 core.

## 5.1 68HC11

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| MC68HC11F1 | PLCC | 68 | 1 KB | 0 (external) | ADC, PWM, UART, I2C, SPI | ROM = 0: this part runs code from external memory (expanded-bus mode) rather than internal Flash. |

## 5.2 HCS08

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| MC9S08SH8 | DIP | 20 | 512 B | 8 KB | ADC, PWM, UART, I2C, SPI | Small 20-pin HCS08 entry point. |
| MC9S08AC60 | QFP | 64 | 2 KB | 59.8 KB | ADC, PWM, UART, I2C, SPI | Largest HCS08 in the set, 64-pin with ~60 KB Flash. |

# 6. Intel MCS-51 Architecture (8051 and Clones)

The classic Intel 8051 core and its many industry clones (Atmel/STC), still widely used for low-cost designs.

## 6.1 Generic / Atmel 8051

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AT89C51 | DIP | 40 | 128 B | 4 KB | UART | Classic original 8051 clone; UART only, no on-chip ADC/PWM. |
| AT89C51RD2 | LQFP | 44 | 2 KB | 64 KB | PWM, UART | Enhanced Atmel 8051 with much larger RAM/Flash than the base AT89C51. |

## 6.2 STC 8051 Clones

Chinese STC clones of the 8051 core, popular for their low cost and built-in in-system programming.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| STC89C58RD+ | DIP | 40 | 1.3 KB | 60 KB | UART | High-Flash 8051 clone, UART-only peripheral set. |
| STC12C5A60S2 | DIP | 40 | 1.3 KB | 60 KB | ADC, PWM, UART | STC's enhanced-core clone, adds ADC and PWM over the 89C58. |

# 7. NXP Industrial 8051-Based Controller

NXP's own 8051-derived part, aimed at industrial PLC-style designs (as reflected in the part's own "_PLC" suffix in the database).

## 7.1 NXP 80C51

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| P87C52X2_PLC (NXP) | PLCC | 44 | 256 B | 8 KB | — | No peripheral flags set in the database; used as a plain industrial 8051-family target. |

# 8. Infineon/STMicroelectronics ST10 (16-bit)

16-bit industrial architecture historically used in automotive and motor-control applications; the only family in the set with native CAN alongside a full peripheral set.

## 8.1 ST10

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ST10F276Z5Q3 | TQFP | 144 | 68 KB | 832 KB | ADC, PWM, UART, I2C, SPI, CAN | By far the largest and most capable chip in the entire database (144 pins, ~68 KB RAM, ~832 KB Flash, CAN bus). |

# 9. Holtek HT4x / HT6x (8-bit)

Holtek's low-cost 8-bit families, common in cost-sensitive consumer electronics. Grouped here by part-number prefix, which identifies the sub-architecture.

## 9.1 HT45 series

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| HT45F4852 | DIP | 28 | 512 B | 16 KB | ADC | Largest Holtek part in the set (Flash-wise). |

## 9.2 HT48 series

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| HT48F06E | DIP | 18 | 64 B | 1 KB | — | Small Flash-based HT48 part, no ADC. |
| HT48R065B | SKDIP | 28 | 96 B | 2 KB | — | OTP (one-time programmable) part — cannot be reflashed after programming. |

## 9.3 HT66 series

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| HT66F018 | DIP | 20 | 192 B | 4 KB | — | No ADC on this part. |
| HT66F50 | DIP | 20 | 256 B | 8 KB | ADC | Largest HT66 in the set, with analog input. |

# 10. Espressif ESP8266 / ESP32 (Wi-Fi SoC)

Wi-Fi-enabled system-on-chip parts. Used in LadderStudio either as the main "mini-PLC" core (talking to an I2C I/O expander) or as a serial communication bridge/co-processor next to another MCU.

## 10.1 ESP8266 — Main core (family: ESP)

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ESP-01 | DIP | 8 | 80 KB | 1 MB | PWM, UART, I2C, Wi-Fi | Used as a mini-PLC core: I/O expansion via an external I2C expander (e.g., MCP23017), since only 2 usable GPIOs are exposed on this module. |
| ESP12F | DIP | 16 | 80 KB | 4 MB | ADC, PWM, UART, I2C, SPI, Wi-Fi | Full-peripheral ESP8266 module with far more usable GPIOs than the ESP-01. |

## 10.2 ESP32 — Main core (family: ESP)

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ESP32-WROOM | DIP | 38 | 520 KB | 4 MB | ADC, PWM, UART, I2C, SPI, Wi-Fi | Dual-core, highest clock (240 MHz) and largest RAM of any chip in the database. |

## 10.3 ESP8266 — Serial/Communication Bridge (family: ESPB)

Same silicon as the ESP-01 above, but configured in the database as a companion co-processor rather than the main ladder-logic core.

| Chip | Package | Pins | RAM | Flash | Peripherals | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ESP-01 (IHM) | DIP | 8 | 80 KB | 1 MB | UART, Wi-Fi | Bridge role: exposes a Wi-Fi-connected HMI (web page/app) to the main MCU over serial. |
| ESP-01 (MQTT) | DIP | 8 | 80 KB | 1 MB | UART, Wi-Fi | Bridge role: relays MQTT publish/subscribe traffic to the main MCU over serial. |

# Appendix — External EEPROM Memory Chips

These are not code-execution targets but external non-volatile memory chips supported by the EEP_WRITE / EEP_READ ladder blocks for persisting variables. Two interface families are supported: I2C (24Cxx series) and SPI (25LCxx series). Parts with 4 KB of capacity or more use 16-bit internal addressing; smaller parts use 8-bit addressing.

| Part | Interface | Capacity | Addressing |
| --- | --- | --- | --- |
| 24C01 | I2C | 128 B | 8-bit |
| 24C02 | I2C | 256 B | 8-bit |
| 24C04 | I2C | 512 B | 8-bit |
| 24C08 | I2C | 1 KB | 8-bit |
| 24C16 | I2C | 2 KB | 8-bit |
| 24C32 | I2C | 4 KB | 16-bit |
| 24C64 | I2C | 8 KB | 16-bit |
| 24C128 | I2C | 16 KB | 16-bit |
| 24C256 | I2C | 32 KB | 16-bit |
| 24C512 | I2C | 64 KB | 16-bit |
| 25LC010 | SPI | 128 B | 8-bit |
| 25LC020 | SPI | 256 B | 8-bit |
| 25LC040 | SPI | 512 B | 8-bit |
| 25LC080 | SPI | 1 KB | 8-bit |
| 25LC160 | SPI | 2 KB | 8-bit |
| 25LC320 | SPI | 4 KB | 16-bit |
| 25LC640 | SPI | 8 KB | 16-bit |
| 25LC128 | SPI | 16 KB | 16-bit |
| 25LC256 | SPI | 32 KB | 16-bit |
| 25LC512 | SPI | 64 KB | 16-bit |
| 25LC1024 | SPI | 128 KB | 16-bit |